# Strelka CAN Testing Notes

Strelka V2:

It appears that Strelka V2 is transmitting CAN messages correctly. It should be noted however that they ONLY work when 5v is connected. They do not work at all if only the STLink is connected, this is because the 5v bus is not powered.

CANH

CANL

I made an interesting discovery being that the MCP2551 CAN transceiver pulls the RX pin high to ~5V. Initially, I thought this would fry the pin of the STM32H743 and STM32F407 (Strelka motor controller) however, after further inspection, it appears that these pins are 5v tolerant according to the datasheet.

STM32F407:

STM32H743:

Note the FT means 5v tolerant:

Strelka in loopback mode:

Strelka works in loopback mode. The transmitter can transmit from the tx fifo without errors and the rx callback occurs straight after.

Testing with CANable transceiver:

Testing with the CANable transceiver showed that the CAN bus on Strelka is working correctly. I learned that for the filter configuration, setting sFilterConfig.FilterType to FDCAN_FILTER_RANGE means that the filter will accept all incoming packets addressed between sFilterConfig.FilterID1and sFilterConfig.FilterID2.

Likewise, the TxHeader1.Identifier field defines the address of the transmitting device.

Strelka Motor Controller:

With a scope on the TXD line, the voltage sits constant at 3.3V and does not change at all. No packets seem to be transmitted. On Strelka, both the TXD and RXD lines periodically transmit data. What is super odd is that data is received on Strelka as if it were being transmitted from the motor controller. I’m not sure how this data has appeared on the RXD line on Strelka if it wasn’t transmitted by the motor controller.

Orange is RXD on motor controller and blue is RXD on Strelka. This strongly suggests that one of them is wired backwards… nope, switching the RXD and TXD lines on the motor controller using the UART switch did not work.

Motor controller TXD pin:

Motor controller TXD pin when STM32 is reset is at ~4.7V. It seems be pulled up by the transceiver and pulled down a bit by the stm32.

Motor controller RXD pin. It periodically goes flat:

Strelka TXD pin. It goes flat periodically:

Strelka RXD pin. Goes flat periodically:

Crazily, powering down the motor controller does not stop signals on the RXD and TXD pins of Strelka.

RXD and TXD of Strelka with motor controller powered off:

Orange is TXD blue is RXD.

RXD and TXD of Strelka connected to CANable device and working correctly:

Orange is TXD blue is RXD.

Strelka motor controller with nothing connected on the other side:

Orange is TXD blue is RXD.

Streka motor controller with CANable connected:

Orange is TXD blue is RXD.

Differential signals with motor controller connected to CANable. Centre voltage is 2.5V:

Strelka connected to CANable. Centre voltage is 2.5V:

The cause of the ringing on the CAN lines could be due to no capacitor. This is the Strelka schematic:

This is the motor controller schematic:

Reading CAN messages from Motor Controller using logic analyser:

This is the format of the sent data:

This is what the logic analyser read:

Motor controller:

This shows correct ID, DLC, byte 0 and byte 1.

*Note, the signal edges are a bit flatter than Strelka’s. I tried shorting the RS pin and it appeared to steepen the edges dramatically. This didn’t cause an ACK however.

Settings used to read this on the logic analyser:

Strelka transmitting an identical frame with no CANable device on the other end (nothing to ACK the packets):

Strelka:

With the CANable device connected:

You can see the ACK bit has been set (low).

Here’s a whole Strelka frame with ACK:

Bus rate decreased to 125kbps due to large number of errors on Motor Controller Bus

Strelka with ACK

Motor controller with no ACK

Strelka connected to CANable with correct ACK:

Pink is Tx, Green is Rx

This shows what a correct packet should look like on the RX TX pins. You can see that the ACK is returned on the Rx pin (but not the Tx as expected).

Motor controller with CANable without ACK:

Pink is Tx, Green is Rx

This does not show an ACK on the Rx line.